Install required tool chain first:  All latest tool chain 2026

1. ARM GNU Toolchain

- Purpose: Compile firmware for RP2040 (Pico/Pico W).

- Recommended version for Pico SDK 2.2.0: arm-gnu-toolchain-14.2.rel1-mingw-w64-i686-arm-none-eabi.exe

- Install notes: Add the bin folder to your PATH if needed (or CMake will detect automatically in Developer Command Prompt).

2. CMake

- Purpose: Generate build files (Ninja or MSVC).

- Recommended: Latest stable (>=3.13).

- Install notes: Add to PATH (default in Windows installer).

cmake –version

3. Ninja

- Purpose: Fast, cross-platform build system used by Pico SDK.

- Install command:

winget install Ninja-build.Ninja

- Verify:

ninja --version

4. Git

- Purpose: Clone SDK and PicoTool repositories.

- Install notes: Use Git for Windows, add git to PATH.

git --version

5. Visual Studio Community (Windows)

- Purpose: Build host tools (picotool.exe, pioasm.exe) using MSVC.

- Workload required:

  - Desktop development with C++

  - Optional: CMake support included

- Why: Pico W projects require some host tools (C++ executables) compiled for Windows.

6. Pico SDK

- Purpose: Core libraries and headers for RP2040 / Pico W projects.

- Clone repo:

git clone -b master https://github.com/raspberrypi/pico-sdk.git C:\Users\soume\Documents\Pico\pico-sdk

- Set environment variable

setx PICO_SDK_PATH "C:\Users\soume\Documents\Pico\pico-sdk"

- Verify:

echo %PICO_SDK_PATH%

7. PicoTool

- Purpose: Convert .elf to .uf2 and other post-processing of binaries.

- Options:

  - Download pre-built release from GitHub

    - PicoTool Releases

  - Build from source using MSVC + Ninja + CMake (required if you need USB support or latest version).

- Build from source:

cd C:\Users\soume\Documents\Pico\picotool

mkdir build && cd build

cmake -G Ninja -DPICO_SDK_PATH=C:\Users\soume\Documents\Pico\pico-sdk ..

ninja

✅ Summary Workflow

- Install ARM GCC toolchain

- Install CMake

- Install Ninja

- Install Git

- Install Visual Studio Community + C++ workload

- Clone Pico SDK and set PICO_SDK_PATH

- Clone or download PicoTool

- Create project folder with CMakeLists.txt and main.c

- Build project (Developer Command Prompt recommended for Pico W):

cmake -G Ninja -DPICO_BOARD=pico_w ..

ninja

- Convert .elf → .uf2 using picotool if you skipped pico_add_extra_outputs()

1)

2)

3)

4)

5)

6)

7) download latest tag version pico tools from github, Release 2.2.0-a4 · raspberrypi/picotool  dowload . or clone it then buils using msvc tool chain, this not build using arm toll chain and not nomal cmd , only builf from developer command prompts